PG 287

DRAFT MANDATE TO CEN

CONCERNING THE EXECUTION OF HARMONISATION WORK

FOR HARMONIZED STANDARDS ON

FIRE PROTECTIVE PRODUCTS

COVERING THE FOLLOWING SUBFAMILIES

1.	REACTIVE (INTUMESCENT) COATINGS FOR FERROUS SUBSTRATES

2. 	FIRE STOPPING AND FIRE SEALING PRODUCTS FOR PENETRATION SEALS, LINEAR JOINT SEALS, AIR TRANSFER GRILLES AND CAVITY BARRIERS

3.	OTHER FIRE PROTECTIVE PRODUCTS: BOARDS / SLABS, MATS, RENDERINGS

FOREWORD

This mandate is issued by the European Commission on the basis of Regulation (EU) 305/2011, hereafter referred to as “the CPR” within the framework of the general policy of the Commission concerning technical harmonisation and standardisation in order to launch the procedure for developing harmonised European standards for certain fire protective products

One of the aims of the CPR is the removal of technical barriers to trade in construction products, in so far as they cannot be removed by means of mutual recognition. Therefore the mandate covers the above construction products in order to remove technical barriers to trade and to enhance their free movement in the internal market.

The mandate defines specific provisions for the development and the quality of the harmonized European standards to allow the declaration of performance of the corresponding construction products.

Pursuant to Articles 2(11) and 17(3) of the CPR, as well as Articles 2(1)(c) and 2(4)(d) of Regulation (EU) 1025/2012 harmonised standards for construction products are to lay down in particular the methods and the criteria for assessing their performance in relation to their essential characteristics. In addition, in accordance with Article 17(4) of the CPR, they are to contain technical details necessary for the implementation of the system of assessment and verification of constancy of performance (the AVCP system).

CHAPTER I: GROUNDS

- This mandate is issued by the European Commission to CEN on the basis of Art. 17(1) and Art. 19(4) of the Regulation (EU) 305/2011 and Art. 10(1) of Regulation (EU) 1025/2012.

- The mandate serves to ensure the quality of harmonized standards for products, which shall reflect the state of the art, with particular reference to the construction products and their corresponding intended use as listed in Annex 1 of the present mandate. The harmonized standards resulting from this mandate must allow the assessment of the performance of the corresponding products.

- The present mandate replaces any previous mandates on the same products previously issued by the European Commission.

CHAPTER II: EXECUTION OF THE MANDATE

- CEN shall, in line with Article 10(3) of Regulation (EU) 1025/2012, indicate within one month following the receipt of this mandate if it accepts the mandate.

- CEN will provide to the Commission the information according to the provisions of Article 10(5) of Regulation (EU) 1025/2012. The information shall be in the form of a work programme and shall include all aspects considered necessary to ensure the quality of the standards in order to allow the appropriate drawing up of the declaration of performance (in accordance with Article 4 of the CPR) of the products covered by the mandate. In particular it will contain the following:

a) the title of the standard;

b) the content of the standard, including reference to those items mentioned in Chapter III, clause 2;

c) the list of supporting documents (national standards, ISO standards, prENs, ENs, ETAGs, EADs, research results, etc.) which might be used in the harmonized standards and indications of those documents that have to be developed by CEN;

d) the timetable for the development of the standard and its submission to the EC, and

e) the identification of the Technical Committee / Working Group responsible;

- Clear differentiation shall be made between the item to become the harmonized standard for the product or product family and the items to be used as supporting documents.

- For the elaboration of harmonised standards for reactive (intumescent) coatings for ferrous substrates, the responsible CEN Technical Committee is requested to consider the already existing ETA Guideline 018 Part 1 and Part 2 on reactive coatings elaborated by EOTA. For the elaboration of the harmonised standards for fire stopping and fire sealing products the responsible CEN Technical Committee is requested to consider the already existing ETA Guideline 026, Parts 1 to 5 on various fire stopping and fire sealing products elaborated by EOTA. For the elaboration of the harmonised standards for other fire protective products the responsible CEN Technical Committee is requested to consider the already existing ETA Guideline 018 Part 1, Part 3 and Part 4 on fire protective boards, slabs, mats and on renderings elaborated by EOTA. Concerning product assessment, the CEN Technical Committee is not obliged to follow the said ETA Guideline, only if this can be justified by the consolidated knowledge of the products and experience. For all points concerning product assessment for which the CEN Technical Committee intends not to follow the said ETA Guideline, the CEN Technical Committee shall timely send to the Commission a detailed justification.

- Where practicable, CEN will make reference to harmonized methods of assessment. When a supporting test method for a characteristic does not exist or is not in the work programme of the CEN TC, a clear statement shall be presented indicating whether CEN is able to produce one or not.

- Any proposals for the addition of products, intended uses, materials and forms not included in the mandate but considered relevant by the CEN TC shall be presented separately from the work programme for further analysis by the Commission services. The CEN TC shall take into account that for all products included in the mandate a system of assessment and verification of constancy of performance is foreseen as defined by the relevant Commission legal act.

Any proposal for the addition of characteristics and durability aspects not included in the mandate but considered relevant by the CEN TC shall be proposed in a special chapter of the work programme for further analysis by the Commission services. The Commission services shall inform the CEN TC if they accept the proposal.

- Levels or classes of requirements for construction works are under the responsibility of Member States and are not covered by the present mandate. As a consequence, they are not expected to be defined in the harmonized standards.

- Levels or classes of performance in relation to the essential characteristics of construction products may be determined as foreseen in the CPR.

Reaction to fire and resistance to fire are product performance aspects for which the need for a classification has been identified and a European classification system has been established. CEN shall use the said classification systems in the harmonised standards in line with Art 27(2) of the CPR.

If the CEN TC identifies a need to include in the harmonised standard a classification, or a minimum/maximum level of performance for other aspects of the product performance it shall immediately inform the Commission thereof providing all details and justifications to allow the Commission to decide on the follow up.

- CEN Technical Committees must give a technical answer for the determination of the essential product characteristics defined by the mandate taking into account the conditions stated below. Test, calculation or other assessment methods suggested must be directly related to the characteristic required and must not make reference to determination methods for characteristics not required by the mandate. Durability requirements shall be dealt with in the framework provided by the current state of the art.

- Reference to test, calculation or other assessment methods must be in accordance with the harmonisation aimed at. In general, only one method shall be referred to for the determination of each characteristic, for a given product or family of products.

- If, however, for a product or family of products because of justifiable reasons, more than one method is to be referred to for the determination of the same characteristic, the situation must be justified. In this case the harmonised standard shall indicate that all referenced test/assessment/calculation methods can be used alternatively.

- Two or more test/assessment/calculation methods for the determination of one characteristic can be accepted only if a correlation between them exists or can be developed. The relevant harmonized product standard must then select one of them as the method of reference.

- Test, calculation or other assessment methods shall have, whenever possible, a horizontal character covering the widest possible range of products.

- Within the work programme, CEN will also specify those cases where the performance-based approach will not be followed in the harmonized standard and will give the relevant justification.

- After examination of the work programme and consultations with CEN, the Commission services may endorse the timetable and the list of standards which meet the terms of this mandate and which will be recognised as harmonized standards, as well as the list of supporting standards where relevant.

- The terms of reference of this mandate may be subject to possible modification or addition, if necessary. Acceptance of the work programme by the Commission services does not imply acceptance of all the items listed as supporting documents. CEN TCs will need to demonstrate the direct link between items for harmonisation and the products, intended uses and characteristics given in the mandate. Nor does acceptance exclude the possibility for further items to be added by CEN in order to fully respond to the terms of the mandate.

- Representatives of the authorities responsible for national regulations have the right and shall be able to participate in the activities of CEN through their national delegations and to present their points of view at all stages of the drafting process of the harmonised standards.

- The Commission may participate in the drafting process as observer and has the right to receive all relevant documents.

- CEN will immediately inform the Commission of any problem relating to the carrying out of the mandate from within the TCs and will present an annual progress report on work within the framework of the mandate.

- The progress report will include a description of work carried out and information on any difficulties being met, whether political or technical, with particular reference to those that might lead the authorities of a Member State to raise objections or to resort to Article 18 of the CPR.

- The progress report will be accompanied by the latest drafts of the standard under the mandate and by updated reports on any subcontracted work.

- Acceptance by CEN of this mandate starts the standstill period referred to in Article 7 of the Directive 98/34/EC of 22 June 1998 (OJEU, L 204 of 21 July 1998, page 37), as modified by article 26 of Regulation (EU) 1025/2012.

- CEN will develop the draft harmonized standards and the relevant supporting standards on the basis of the work programme and will inform the Commission in good time that the draft is being circulated for public enquiry.

- CEN will present the established harmonized standards to the Commission to allow for the assessment of their conformity with the mandate (Art. 17(5) of the CPR.

- CEN members will publish the standards transposing the harmonized European standards at the latest 6 months after a positive vote in CEN. National standards covering the same scope will continue to be applicable until the end of the coexistence period defined for the relevant harmonised standards (Article 17(5) of the CPR).

CHAPTER III: HARMONIZED STANDARDS

- Harmonized standards shall be prepared allowing the expression in a common European way of the performance of the products for the intended uses listed in Annex 1 of the mandate and in relation to their essential characteristics listed in Annex 2. One of the purposes of the CPR being to remove barriers to trade, the standards deriving from it will therefore be expressed, as far as practicable in product performance terms.

- The harmonized standard shall contain:

- 	A detailed scope and field of application;

- 	A detailed description of the product or family of products covered and the relevant intended uses of the different products;

- 	The definition of the essential characteristics of the products listed in Annex 2 of the mandate (expressed in performance terms, as far as practicable) that are relevant to the satisfaction of the basic requirements for construction works taking into account the intended use;

- 	The methods (calculation, test methods or others) or a reference to a standard containing the methods for the determination of such characteristics;

- 	The classification system(s) and the levels of performances, if required by the mandate;

- 	The system for assessing and verifying the constancy of performance as required in Annex 3 of the mandate and the corresponding specific provisions for the assessment and verification of the constancy of performance.

- As far as possible, each standard will make reference to performances common to other standards developed under mandate and which constitutes a cohesive and compatible group of harmonized European standards developed in parallel. CEN shall ensure consistency within the whole package.

- Harmonized standards must permit construction products which allow works to meet the basic requirements and which are produced and used lawfully in accordance with technical traditions warranted by local climatic and other conditions to continue to be placed on the market.

- The basic requirements being expressed in terms of performance of the works, the characteristics of the products shall be also expressed in terms of performance so that, in referring to the harmonized European standards, the regulations may "approximate" evolving in terms of "performance requirements". As far as practicable and depending on the intended use mentioned in the Annexes of this mandate, the harmonised standard shall include a definition of the durability in term of performance of the declared values of the performances of product characteristics as well as suitable methods for its evaluation against the actions listed in Annex 3 of the mandate.

- The relevant systems for the assessment and verification of constancy of performance, according to Article 28 of and Annex V to the CPR, are listed in Annex 3 of the mandate. For the establishment of the corresponding specific provisions for the assessment and verification of constancy of performance, the harmonized standard will take into account:

- 	the different intended uses of the product mentioned in the Annexes to the present mandate and, if any, the different levels or classes of performance;

- 	cases of individual (non-series) production according to Article 38 of the CPR;

- 	the provisions of paragraph 3 of Annex 3 to the present mandate.

ANNEX 1

FIELD OF APPLICATION

1)  REACTIVE (INTUMESCENT) COATINGS FOR FERROUS SUBSTRATES

INTENDED TO BE USED IN:

-  fire protection of ferrous structural elements

List of products intended to be covered by the mandate

| FORM | MATERIALS | PRODUCTS FOR CONSIDERATION |
| --- | --- | --- |
| Liquid coatings or mastic | e.g.: - Acid source - Binder - Plasticiser - Blowing agent - Reinforcement fibre - Carbon source - Liquid carrier/ coalescing solvent - Organic solvent - Mineral filler - Titanium dioxide - Wetting and dispersion agent - Thixotrope | e.g. water borne, solvent borne, solvent-free reactive coatings, mastics and kits for the fire protection of ferrous structural elements |

2) FIRE STOPPING AND FIRE SEALING PRODUCTS FOR PENETRATION SEALS, LINEAR JOINT SEALS, AIR TRANSFER GRILLES AND CAVITY BARRIERS

INTENDED TO BE USED IN:

04/33	EXTERNAL WALLS (INCLUDING CLADDING AND CURTAIN WALLING), INTERNAL WALLS AND PARTITIONS

05/33	Floors, galleries and ceilingS

06/33 PREFABRICATED SYSTEMS FOR FLOORS AND GALLERIES, STAIRS, RAMPS, RAISED ACCESS FLOORS, BALUSTRADES AND HAND RAILS, INCLUDING EXTERNAL WORKS

07/33	Roofs

10/33	Suspended ceilings

18/33	Drainage and disposal of other liquids and gaseous waste

19/33	Supply of hot and cold water

20/33	Supply of fuels, oils and other liquids

22/33	Supply of gases, pressure and vacuum systems

23/33	Space heating, cooling and air conditioning

24/33	Supply of electricity

26/33	Communications

List of products intended to be covered by the mandate

2.1  FIRE STOPPING AND FIRE SEALING PRODUCTS/KITS FOR PENETRATION SEALS

| FORM | MATERIALS | PRODUCTS FOR CONSIDERATION |
| --- | --- | --- |
| Flexible Rigid Formless | e.g. CaSi Chemicals Composite materials Gypsum, Mortar, Sand Metal Mineral wool Plastics Polymers (e.g Foams) Reactive / expanding materials Rubber Textiles | e.g. Bags, Pillows, Cushions Bellows Blank, coated or faced sheets/boards/mats/slabs Blocks, Plugs Intumescent/ablative/insulating coatings Membrane forming sprays Modular (Transit) Systems Pre-formed pipe closure devices /cable seal devices Putty pads,-sticks,-strips Sand gaskets Sealants, Mastics, Foams Sleeves Wraps, Bandages, Collars Kits of the above |

2.2  FIRE STOPPING AND FIRE SEALING PRODUCTS/KITS FOR LINEAR JOINT SEALS

| FORM | MATERIALS | PRODUCTS FOR CONSIDERATION |
| --- | --- | --- |
| Flexible Rigid Formless | e.g. Composite materials Metal Mineral Wool Plastics Polymers (e.g. Foam) Reactive / expanding materials Rubber Textiles (e.g. Glass fibre) | e.g. Bags, Pillows, Cushion Bellows Blank, coated or faced sheets/boards/mats Blocks Intumescent/ablative coatings Membrane-forming sprays Pre-formed cords, strips Putty pads,-sticks,-strips Sealants, Mastics, Foams Kits of the above |

2.3  FIRE STOPPING AND FIRE SEALING PRODUCTS/KITS FOR AIR TRANSFER GRILLES

| FORM | MATERIALS | PRODUCTS FOR CONSIDERATION |
| --- | --- | --- |
| Flexible, Rigid | e.g. Metal Plastics Chemicals Reactive / expanding materials | e.g. Grilles Kits |

2.4  FIRE STOPPING AND FIRE SEALING PRODUCTS/KITS FOR CAVITY BARRIERS

| FORM | MATERIALS | PRODUCTS FOR CONSIDERATION |
| --- | --- | --- |
| Flexible Rigid Systems | e.g. Mineral Wool Plastics Polymers Textiles (e.g. glass fibre) Composite materials Chemicals Reactive / expanding materials | e.g. Curtains, e.g. made of coated or faced fabrics Mineral wool batts, usually encapsulated Blank, coated or faced sheets/boards/mats/slabs Strips Pillows Kits of the above |

3) OTHER FIRE PROTECTIVE PRODUCTS: BOARDS / SLABS, MATS, RENDERINGS

INTENDED TO BE USED IN:

04/33	EXTERNAL WALLS (INCLUDING CLADDING AND CURTAIN WALLING), INTERNAL WALLS AND PARTITIONS

05/33	Floors, galleries and ceilingS

06/33	PREFABRICATED SYSTEMS FOR FLOORS AND GALLERIES, STAIRS, RAMPS, RAISED ACCESS FLOORS, BALUSTRADES AND HAND RAILS, INCLUDING EXTERNAL WORKS

07/33	Roofs

08/33	Frames

09/33	EXTERNAL AND INTERNAL DOORS AND WINDOWS, ROOF OPENINGS AND ROOFLIGHTS (INCLUDING FIRE DOORS AND SHUTTERS)

10/33	Suspended ceilings

11/33 	EXTERNAL FINISHES OF WALLS

12/33 	INTERNAL FINISHES OF WALLS AND PARTITIONS

13/33 	FLOOR AND STAIR FINISHES

14/33 	CEILING FINISHES

18/33 	Drainage and disposal of other liquids and gaseous waste

19/33 	Supply of hot and cold water

20/33 	Supply of fuels, oils and other liquids

21/33 	FIRE SUPPRESSION AND EXTINGUISHING SYSTEMS

22/33 	Supply of gases, pressure and vacuum systems

23/33 	Space heating, cooling and air conditioning

24/33 	Supply of electricity

26/33 	Communications

28/33 	FIRE DETECTION AND ALARM

List of products intended to be covered by the mandate

| FORM | MATERIALS | PRODUCTS FOR CONSIDERATION |
| --- | --- | --- |
| Boards / slabs (Rigid panels with large or small size) | e.g. Calcium Silicate Metal Mineral wool Vermiculite (Expanded) perlite Gypsum/Cementitious materials | Boards/slabs(1), e.g.: - Fire protective CaSi boards or slabs, - Fire protective Mineral wool boards or slabs - Fire protective Gypsum boards and blocks - Fire protective Composite boards or slabs, blank or coated or faced - Fire protective Vermiculite boards / slabs. - Kits of the above boards/slabs and fixing accessories |
| Mats (flexible sheets/rolls) | Composite, Mineral wool Inorganic fabrics Gypsum / Cementitious materials with aggregates/fibre reinforcement | Mats, e.g.: - Fire protective mineral wool mats, faced or not - Wrapping fabrics - Kits of mats, fabrics and fixing accessories |
| Renderings | Fibrous materials Metal Fixings Paints | Renderings, e.g.: - Gypsum based sprays and trowel grades - Cement based sprays and trowel grades - Mineral fibre sprays mixed with binders/aggregates - Fixings - Rendering kits |

(1) Excluding boards and slabs for fire protection of walls or ceilings and suspended ceiling kits as included in the mandate on “internal and external wall and ceiling finishes”

ANNEX 2

TECHNICAL TERMS OF REFERENCE

1)  REACTIVE (INTUMESCENT) COATINGS FOR FERROUS SUBSTRATES

Description of the family:

Products/kits specifically formulated to provide physical and/or chemical reaction upon heating such that their physical and/or chemical form change and in so doing provide fire protection by thermal insulating and cooling effect.

Reactive coatings may be applied directly to the substrate or over a priming system. They also may require a protective top-coat, if in-service conditions, such as high temperature or high humidity, may degrade the performance of the reactive coating.

Essential Characteristics of Reactive Coatings to be covered by the harmonized standard are:

| BRCW | PERFORMANCE CHARACTERISTIC | Durability |
| --- | --- | --- |
| 1 |  | Y (Against corrosion, different environmental conditions, chemicals, biological attack) |
| 2 | Resistance to fire Reaction to fire |  |
| 3 | Air permeability Water permeability Release of dangerous substances |  |
| 4 | Adhesion |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |

2. FIRE STOPPING AND FIRE SEALING PRODUCTS FOR PENETRATION SEALS, LINEAR JOINT SEALS, AIR TRANSFER GRILLES AND CAVITY BARRIERS

2.1 FIRE STOPPING AND FIRE SEALING PRODUCTS/KITS FOR PENETRATION SEALS

Description of the family:

Products, including kits, intended to prevent or restrict the passage of fire and/or smoke and/or to maintain the integrity and the insulation performance of a separating element where services pass through. The services passing through may be e.g. cables, cable carriers, bus bars, bus bar trunking units, pipes and conduits.

Products included in this mandate are e.g. Bags, Pillows, Cushions, Bellows, Blank, coated or faced sheets/boards/mats/slabs, Blocks, Plugs, Intumescent/ablative/insulating coatings, Membrane forming sprays, Modular (Transit) Systems, Pre-formed pipe closure devices /cable seal devices, Putty pads,-sticks,-strips, Sand gaskets, Sealants, Mastics, Foams, Sleeves., Wraps, Bandages, Collars, kits of the above.

Products may be used as single product or in combinations e.g.:

- Bags, Pillows, Cushions  + ancillary products (Sealants, Mastics, Foams, Sleeves, Wraps, Bandages, Collars, Pre-formed pipe closure devices /cable seal devices , Putty pads,-sticks,-strips)

- Intumescent/ablative/insulating coatings + ancillary products (Sealants, Mastics, Foams, Sleeves, Wraps, Bandages, Collars , Pre-formed pipe closure devices /cable seal devices, Putty pads,-sticks,-strips)

-Blocks, Plugs + ancillary products (Sealants, Mastics, Foams, Sleeves, Wraps, Bandages, Collars, Pre-formed pipe closure devices /cable seal devices, Putty pads,-sticks,-strips)

-Foams, Sealants, Mastics, Foams, Sleeves, Wraps, Bandages, Collars, Pre-formed pipe closure devices /cable seal devices, Putty pads,-sticks,-strips)

Essential Characteristics for Fire Stopping and Fire Sealing Products (single or in combinations) for penetration seals to be covered by the harmonized standard are:

| BRCW | PERFORMANCE CHARACTERISTIC | Durability |
| --- | --- | --- |
| 1 |  | Y (Against different environmental conditions, as relevant) |
| 2 | Resistance to fire Reaction to fire |  |
| 3 | Air/Gas permeability (not relevant for products which are installed outside a building element and a gap is left between the service and the building element and/or product) - Water tightness (not relevant for products which are installed outside a building element and a gap is left between the service and the building element and/or product) - Release of dangerous substances |  |
| 4 | Resistance to impact Movement capability Adhesion Electrical resistance |  |
| 5 | Airborne sound insulation Structure borne sound decoupling (only relevant for products intended to form a penetration seal around pipework) |  |
| 6 | Water vapour permeability Thermal insulation |  |
| 7 |  |  |

Cavity barriers it of the roof Told smoke and hot gases whilst providing a path for air movement in normal conditions.onsible f

2.2 FIRE STOPPING AND FIRE SEALING PRODUCTS FOR LINEAR JOINT SEALS:

Description of the family:

Products (including kits of components) intended to prevent or restrict the passage of fire and/or smoke between elements or components or to maintain the integrity and insulation performance of one or more fire separating elements at linear discontinuities.

Excluded are: Sealants for glazing, sanitary joints, pedestrian walkways and for external/internal walls/floors not intended to be used to prevent or restrict the passage of fire and/or smoke.

Essential Characteristics for Fire Stopping and Fire Sealing products for linear joint seals to be covered by the harmonized standards are:

| BRCW | PERFORMANCE CHARACTERISTIC | Durability |
| --- | --- | --- |
| 1 |  | Y (Against different environmental conditions, as relevant) |
| 2 | - Resistance to fire - Reaction to fire |  |
| 3 | Air/Gas permeability Water tightness Release of dangerous substances |  |
| 4 | Air/Gas permeability (not relevant for products which are installed outside a building element and a gap is left between the service and the building element and/or product) - Water tightness (not relevant for products which are installed outside a building element and a gap is left between the service and the building element and/or product) Movement capability (relevant to compensate movement of the substrate) Release of dangerous substances |  |
| 5 | Airborne sound insulation |  |
| 6 | - Water vapour permeability - Thermal insulation |  |
| 7 |  |  |

2.3  FIRE STOPPING AND FIRE SEALING PRODUCTS FOR AIR TRANSFER GRILLES:

Description of the family:

Preformed or pre-assembled products or components sold as a kit, reactive or mechanical, to prevent the spread of fire, cold smoke and hot gases whilst providing a path for air movement in normal conditions.

Essential Characteristics for Fire Stopping and Fire Sealing products for Air Transfer Grilles to be covered by the harmonized standards are:

| BRCW | PERFORMANCE CHARACTERISTIC | Durability |
| --- | --- | --- |
| 1 |  | Y (Against different environmental conditions, as relevant) |
| 2 | - Resistance to fire - Reaction to fire |  |
| 3 | - Air permeability - Water tightness - Release of dangerous substances |  |
| 4 | - Mechanical strength - Adhesion - Cold smoke leakage |  |
| 5 | - Airborne sound insulation |  |
| 6 |  |  |
| 7 |  |  |

2.4  FIRE STOPPING AND FIRE SEALING PRODUCTS FOR CAVITY BARRIERS:

Description of the family:

Preformed or pre-assembled products, or components sold as a kit, intended to be used as non-loadbearing, vertically or horizontally oriented barriers, designed to provide fire separation within a concealed space (e.g. between leaves of double-leaved walls, underneath raised access floors, above suspended ceilings, in loft areas between the ceiling and the underside of the roof) horizontal and/or vertical, or at its edge.

Essential Characteristics for Fire Stopping and Fire Sealing products for Cavity Barriers to be covered by the harmonized standards are:

| BRCW | PERFORMANCE CHARACTERISTIC | Durability |
| --- | --- | --- |
| 1 |  | Y (Against different environmental conditions, as relevant) |
| 2 | - Resistance to fire - Reaction to fire |  |
| 3 | - Air permeability - Release of dangerous substances |  |
| 4 | Mechanical strength Resistance to impact Movement capability |  |
| 5 | - Airborne sound insulation |  |
| 6 | - Water vapour permeability - Thermal resistance |  |
| 7 |  |  |

3) OTHER FIRE PROTECTIVE PRODUCTS:

BOARDS / SLABS, MATS, RENDERINGS

3.1  OTHER FIRE PROTECTIVE PRODUCTS:  BOARDS / SLABS

Description of the family: Components intended to improve fire performance of building elements or to provide fire protection, including fire protection of services. Note: Boards/slabs for the fire protection of walls, ceilings and suspended ceiling kits (as included in the mandate on “internal and external wall and ceiling finishes” are not covered by this mandate).

Essential Characteristics of the fire protective boards/slabs to be covered by the harmonized standards are:

| BRCW | PERFORMANCE CHARACTERISTIC | Durability |
| --- | --- | --- |
| 1 |  | Y (Against different environmental conditions, as relevant) |
| 2 | Resistance to fire Reaction to fire |  |
| 3 | Water permeability Release of dangerous substances |  |
| 4 | Flexural strength Pull out/through resistance of fasteners Resistance to impact Dimensional stability Adhesion |  |
| 5 | Sound absorption Airborne Sound insulation Impact sound insulation |  |
| 6 | Thermal resistance Water vapour permeability |  |
| 7 |  |  |

3.2  OTHER FIRE PROTECTIVE PRODUCTS:  MATS

Description of the family: Products, including kits, intended to improve fire performance of building elements or to provide fire protection, including fire protection of services.

Essential Characteristics of the fire protective mats to be covered by the harmonized standards are:

| BRCW | PERFORMANCE CHARACTERISTIC | Durability |
| --- | --- | --- |
| 1 |  | Y (Against different environmental conditions, as relevant) |
| 2 | Resistance to fire Reaction to fire |  |
| 3 | Water permeability Release of dangerous substances |  |
| 4 | Flexural strength Pull out/through resistance of fasteners Resistance to impact Dimensional stability Adhesion |  |
| 5 | Sound absorption Airborne Sound insulation Impact sound insulation |  |
| 6 | Thermal resistance Water vapour permeability |  |
| 7 |  |  |

3.3   OTHER FIRE PROTECTIVE PRODUCTS:  RENDERINGS

.

Description of the family: Products, including kits, intended to improve fire performance of building elements or to provide fire protection, including fire protection of services applicable to the following substrate materials: metal, concrete, timber, masonry, boards of different materials.

Essential Characteristics of the fire protective renderings to be covered by the harmonized standards are:

| BRCW | PERFORMANCE CHARACTERISTIC | Durability |
| --- | --- | --- |
| 1 |  | Y (Against corrosion, different environmental conditions, as relevant) |
| 2 | Resistance to fire Reaction to fire |  |
| 3 | Water absorption Release of dangerous substances |  |
| 4 | Resistance to impact Flexural performance Adhesion Pull out resistance (fixings, keying mesh) |  |
| 5 | Sound absorption Sound insulation Impact sound insulation |  |
| 6 | Thermal resistance Water vapour permeability |  |
| 7 |  |  |

ANNEX 3

ASSESSMENT AND VERIFICATION OF CONSTANCY OF PERFORMANCE

FIRE PROTECTIVE PRODUCTS (1/2)

1. 	Levels and classes for product performances

Reaction to fire and resistance to fire are the product performance aspects for which the need for a classification has been identified and a European classification system has been established. CEN shall use the said classification systems in the harmonised standards in line with Art 27(2) of the CPR.

Regarding reaction to fire, the relevant classification system is indicated in the Commission Decision 2000/147/EC as amended and CEN is requested to make reference to the standard(s) prepared under Commission mandate to CEN/CENELEC "Horizontal complement to the mandates in respect of reaction to fire" in dealing with reaction to fire in the specific harmonised European standards to be developed under this mandate.

Regarding resistance to fire, the relevant classification system is indicated in the Commission Decision 2000/367/EC as amended and CEN is requested to make reference to the standard(s) prepared under Commission mandate to CEN/CENELEC "Horizontal complement to the mandates in respect of resistance to fire" in dealing with resistance to fire in the specific harmonised European standards to be developed under this mandate.

2. 	Systems of assessment and verification of constancy of performance

For the product(s) and intended use(s) listed below, CEN is requested to specify the following system of assessment and verification of constancy of performance in the relevant harmonized European standards:

| Products | Intended use(s) | Level(s) or class(es) (Resistance to fire) | System of assessment and verification of constancy of performance |
| --- | --- | --- | --- |
| Reactive (intumescent) coatings/kits for ferrous substrates Fire Stopping and Fire Sealing products/kits for Penetration Seals, Linear Joint Seals, Air Transfer Grilles and for Cavity Barriers Other fire protective products/kits: boards / slabs, mats, renderings | for fire compartmentation and/or fire protection | any | 1 |
| System 1: See Regulation (EU) 305/2011, Annex V.1.2 |  |  |  |

3. 	Conditions to be applied by CEN on the specifications of the assessment and verification of constancy of performance

3.1 	The specification for the system should be such that it can be implemented even where performance does not need to be determined for a certain characteristic, because at least one Member State has no legal requirement at all for such characteristic [see Article 6.3(f) of the Regulation (EU) 305/2011]. In those cases the verification of such a characteristic must not be imposed on the manufacturer if he does not wish to declare the performance of the product in that respect.

3.2 	For products under systems 1 and 3, regarding the determination of the product type [see Annex V.1.2 (b) (i) and V.1.4 (b) of the Regulation (EU) 305/2011], the task for the notified body will be limited to the assessment of the following characteristics:

For fire stopping and fire sealing products for Penetration Seals, Linear Joint Seals, Air Transfer Grilles and for Cavity Barriers:

Resistance to fire

Reaction to fire

Mechanical resistance and stability

Release of dangerous substances

Acoustic performance

For other fire protective products (boards/slabs):

Resistance to fire

Flexural strength

Dimensional stability

Resistance to impact

Resistance to movement

For other fire protective products (mats):

Resistance to fire

Compressive strength

Tensile strength

Dimensional stability

For other fire protective products (renderings):

Resistance to fire

Resistance to impact

Adhesion

Flexural performance

3.3 For products under system 1, for the initial inspection of the manufacturing plant and of the factory production control [see Annex V.1.2 (b), (ii) of the Regulation (EU) 305/2011], and for the continuous surveillance, assessment and approval of the factory production control [see Annex V.1.2 (b), (iii) of the Regulation (EU) 305/2011], parameters related to the following characteristics shall be of the interest for the notified body:

For fire stopping and fire sealing products for Penetration Seals, Linear Joint Seals,  Air Transfer Grilles and for Cavity Barriers:

Resistance to fire

Reaction to fire

Mechanical resistance and stability

Release of dangerous substances

Acoustic performance

For fire protective boards/slabs:

Resistance to fire

Flexural strength

Dimensional stability

For fire protective products mats:

Resistance to fire

Tensile strength

Dimensional stability

For fire protective products renderings:

Resistance to fire

Adhesion

FIRE PROTECTIVE PRODUCTS (2/2)

1. 	Levels and classes for product performances

Reaction to fire and resistance to fire are the product performance aspects for which the need for a classification has been identified and a European classification system has been established. CEN shall use the said classification systems in the harmonised standards in line with Art 27(2) of the CPR.

Regarding reaction to fire, the relevant classification system is indicated in the Commission Decision 2000/147/EC as amended and CEN is requested to make reference to the standard(s) prepared under Commission mandate to CEN/CENELEC "Horizontal complement to the mandates in respect of reaction to fire" in dealing with reaction to fire in the specific harmonised European standards to be developed under this mandate.

Regarding resistance to fire, the relevant classification system is indicated in the Commission Decision 2000/367/EC as amended and CEN is requested to make reference to the standard(s) prepared under Commission mandate to CEN/CENELEC "Horizontal complement to the mandates in respect of resistance to fire" in dealing with resistance to fire in the specific harmonised European standards to be developed under this mandate.

2. 	Systems of assessment and verification of constancy of performance

For the product(s) and intended use(s) listed below, CEN is requested to specify the following system of assessment and verification of constancy of performance in the relevant harmonized European standards:

| Product(s) | Intended use(s) | Level(s) or class(es) (Reaction to fire) | System(s) of assessment and verification of constancy of performance |
| --- | --- | --- | --- |
| Reactive (intumescent) coatings/kits for ferrous substrates Fire Stopping and Fire Sealing products/kits for Penetration Seals, Linear Joint Seals, Air Transfer Grilles and for Cavity Barriers Other fire protective products/kits: boards / slabs, mats, renderings | for uses subject to regulations on reaction to fire | A1(1), A2(1), B(1), C(1) | 1 |
|  |  | A1(2), A2(2), B(2), C(2), D, E | 3 |
|  |  | (A1 to E)(3), F | 4 |
| System 1: See Regulation (EU) 305/2011, Annex V.1.2 System 3: See Regulation (EU) 305/2011, Annex V.1.4 System 4: See Regulation (EU) 305/2011, Annex V.1.5 |  |  |  |

(1)  Products/materials for which a clearly identifiable stage in the production process results in an improvement of the reaction to fire classification (e.g. an addition of fire retardants or a limiting of organic material).

(2)    Products/materials not covered by footnote 1.

(3)  	Products/ materials that do not require to be tested for reaction to fire (e.g. Products/materials of Classes A1 according to Commission Decision 96/603/EC).

3. 	Conditions to be applied by CEN on the specifications of the assessment and verification of constancy of performance

3.1 	The specification for the system should be such that it can be implemented even where performance does not need to be determined for a certain characteristic, because at least one Member State has no legal requirement at all for such characteristic [see Article 6.3(f) of the Regulation (EU) 305/2011]. In those cases the verification of such a characteristic must not be imposed on the manufacturer if he does not wish to declare the performance of the product in that respect.

3.2 	For products under systems 1 and 3, regarding the type testing of the product [see Annex V.1.2 (b) (i) and V.1.4 (b) of the Regulation (EU) 305/2011], the task for the notified testing laboratory will be limited to the assessment of the following characteristics:

- Class characteristics for reaction to fire, as indicated in the Commission Decision 94/611/EC as amended by Commission Decision 96/603/EC.

3.3 For products under system 1, for the initial inspection of the manufacturing plant and of the factory production control [see Annex V.1.2 (b), (ii) of the Regulation (EU) 305/2011], and for the continuous surveillance, assessment and approval of the factory production control [see Annex V.1.2 (b), (iii) of the Regulation (EU) 305/2011], parameters related to the following characteristics shall be of the interest of the notified product certification:

- Class characteristics for reaction to fire, as indicated in the Commission Decision 94/611/EC as amended by Commission Decision 96/603/EC.

ANNEX 4

DANGEROUS SUBSTANCES

1.	REACTIVE (INTUMESCENT) COATINGS FOR FERROUS SUBSTRATES

2. 	FIRE STOPPING AND FIRE SEALING PRODUCTS FOR PENETRATION SEALS, LINEAR JOINT SEALS, AIR TRANSFER GRILLES AND CAVITY BARRIERS

3.	OTHER FIRE PROTECTIVE PRODUCTS: BOARDS / SLABS, MATS, RENDERINGS

European technical specifications must be adopted taking into account necessary legislation on substances classified as dangerous.

In addition, outside the scope of the Regulation, writers of technical specifications must take into account legislation which affects materials to be used for construction products and which are regulated for reasons not related to the incorporation of the construction products into the works.

Specification writers will find relevant data on EU and national regulations with regard to Basic Requirement for Construction Works 3 in the Commission's CP-DS database available in http://ec.europa.eu/enterprise/construction/cpdds/, but must also take account of any other relevant legislation on dangerous substances, which the database does not yet include.